NAME:R.THARUN

REG.NO:192412645

DATE: 28/07/2026

Ex.No:1: Simulation of FDMA in MATLAB

Aim:

To simulate a Frequency Division Multiple Access (FDMA) system using MATLAB

and analyze its communication performance in terms of channel bandwidth, data rate, and

signal-to-noise ratio (SNR).

Procedure :

- Open MATLAB and create a new script file.

- Define the total available bandwidth, number of FDMA users, carrier frequencies, signal power, and noise power.

- Allocate a unique frequency channel to each user and calculate the channel bandwidth.

- Compute the data rate for each user based on the allocated bandwidth and modulation scheme.

- Calculate the Signal-to-Noise Ratio (SNR) using the signal and noise power values.

- Plot the transmitted signals of all FDMA users using different carrier frequencies.

- Display the calculated channel bandwidth, data rate, and SNR in the MATLAB Command Window.

- Analyze the results to verify independent channel allocation and evaluate the overall performance of the FDMA system.

Program :

clc; clear; close all;

fs=1000; t=0:1/fs:1;

TBW=600; Users=3;

CBW=TBW/Users;

fc=[100 300 500];

m=sin(2*pi*10*t);

s1=m.*cos(2*pi*fc(1)*t);

s2=m.*cos(2*pi*fc(2)*t);

s3=m.*cos(2*pi*fc(3)*t);

subplot(2,1,1)

plot(t,s1,'r',t,s2,'g',t,s3,'b'),grid on

title('FDMA Channel Allocation'),xlabel('Time (s)'),ylabel('Amplitude')

subplot(2,1,2)

bar(1:Users,CBW*ones(1,Users)),grid on

xlabel('Users'),ylabel('Channel Bandwidth (kHz)'),title('Channel Bandwidth per User')

Output

Objective 2:

Program:

clc; clear; close all;

BW = 200; % Channel Bandwidth (kHz)

Users = 3;

M = 2; % BPSK Modulation

DataRate = BW*log2(M); % Data Rate (kbps)

Rate = DataRate*ones(1,Users);

disp(' User Data Rate (kbps)')

disp([(1:Users)' Rate'])

figure

bar(1:Users,Rate)

xlabel('Users')

ylabel('Data Rate (kbps)')

title('FDMA Data Rate for Each User')

grid on

Output

Objective 3

Program:

clc; clear; close all;

Ps = 1; % Signal Power (W)

Pn = [0.1 0.05 0.01]; % Noise Power (W)

Users = 1:3;

SNR = 10*log10(Ps./Pn); % SNR in dB

disp('User Noise Power(W) SNR(dB)')

disp([Users' Pn' SNR'])

figure

bar(Users,SNR)

xlabel('Users')

ylabel('SNR (dB)')

title('Signal-to-Noise Ratio of FDMA Users')

grid on

Output

Result: The FDMA system was successfully simulated by allocating separate frequency channels to three users, and thechannel bandwidth assigned to each user was determined.